ABEL 4.30  -  Device Utilization Chart         Fri Apr 19 12:28:13 1996

Four phase stepper motor controller Type 3

==== P22V10 Programmed Logic ====

QA.D   = (  !STEP & QA.FB

#   !NEW_STEP & !QC.FB & QA.FB

#   STEP & NEW_STEP & !QA.FB

#   STEP & QC.FB & !QA.FB ); " ISTYPE 'BUFFER'

QA.C   = (  CLOCK );

QB.D   = (  !STEP & QB.FB

#   !NEW_STEP & !QC.FB & QB.FB

#   !DIR & QB.FB & QA.FB

#   STEP & DIR & NEW_STEP & !QB.FB & QA.FB

#   STEP & DIR & QC.FB & !QB.FB & QA.FB

#   DIR & QB.FB & !QA.FB

#   STEP & !DIR & NEW_STEP & !QB.FB & !QA.FB

#   STEP & !DIR & QC.FB & !QB.FB & !QA.FB ); " ISTYPE 'BUFFER'

QB.C   = (  CLOCK );

QC.D   = (  !STEP & CA & CB

#   !STEP & QC.FB

#   CA & CB & !NEW_STEP & !QC.FB ); " ISTYPE 'BUFFER'

QC.C   = (  CLOCK );

CA.D   = (  !STEP & CB.FB

#   !STEP & !CA.FB ); " ISTYPE 'BUFFER'

CA.C   = (  CLOCK );

CB.D   = (  !STEP & CB.FB

#   !STEP & CA.FB ); " ISTYPE 'BUFFER'

CB.C   = (  CLOCK );

NEW_STEP.D   = (  !STEP ); " ISTYPE 'BUFFER'

NEW_STEP.C   = (  CLOCK );

PHASE_4     = (  QB & !QC );

PHASE_3     = (  !QB & !QC );

PHASE_2     = (  !QA & QB & !QC

#   QA & !QB & !QC );

PHASE_1     = (  QA & QB & !QC

#   !QA & !QB & !QC );

Warning 5087: Mapping QA to QA.FB; only one feedback path is allowed on pin 14

Warning 5087: Mapping QB to QB.FB; only one feedback path is allowed on pin 15

Warning 5087: Mapping QC to QC.FB; only one feedback path is allowed on pin 16

Warning 5087: Mapping CA to CA.FB; only one feedback path is allowed on pin 21

Warning 5087: Mapping CB to CB.FB; only one feedback path is allowed on pin 22

Page 2

ABEL 4.30  -  Device Utilization Chart         Fri Apr 19 12:28:16 1996

Four phase stepper motor controller Type 3

==== P22V10 Chip Diagram ====

P22V10

+---------\       /---------+

|          \     /          |

|           -----           |

CLOCK |  1                    24  | Vcc

|                           |

STEP |  2                    23  | NEW_STEP

|                           |

DIR |  3                    22  | CB

|                           |

|  4                    21  | CA

|                           |

|  5                    20  | PHASE_4

|                           |

|  6                    19  | PHASE_3

|                           |

|  7                    18  | PHASE_2

|                           |

|  8                    17  | PHASE_1

|                           |

|  9                    16  | QC

|                           |

| 10                    15  | QB

|                           |

| 11                    14  | QA

|                           |

GND | 12                    13  |

|                           |

|                           |

`---------------------------'

SIGNATURE: N/A

Page 3

ABEL 4.30  -  Device Utilization Chart         Fri Apr 19 12:28:16 1996

Four phase stepper motor controller Type 3

==== P22V10 Resource Allocations ====

Device        | Resource  |   Design    |    Part     |

Resources      | Available | Requirement | Utilization | Unused

======================|===========|=============|=============|==============

|           |             |             |

Dedicated input pins  |     12    |       3     |     3       |     9 ( 75 %)

Combinatorial inputs  |     12    |       3     |     3       |     9 ( 75 %)

Registered inputs     |      -    |       0     |     -       |     -

|           |             |             |

Dedicated output pins |      -    |       4     |     -       |     -

Bidirectional pins    |     10    |       6     |    10       |     0 (  0 %)

Combinatorial outputs |      -    |       4     |     -       |     -

Registered outputs    |      -    |       6     |     -       |     -

Reg/Com outputs       |     10    |       -     |    10       |     0 (  0 %)

Two-input XOR         |      -    |       0     |     -       |     -

|           |             |             |

Buried nodes          |      -    |       0     |     -       |     -

Buried registers      |      -    |       0     |     -       |     -

Buried combinatorials |      -    |       0     |     -       |     -

Page 4

ABEL 4.30  -  Device Utilization Chart         Fri Apr 19 12:28:16 1996

Four phase stepper motor controller Type 3

==== P22V10 Product Terms Distribution ====

Signal             |    Pin   | Terms | Terms | Terms

Name              | Assigned | Used  |  Max  | Unused

===============================|==========|=======|=======|=======

QA.REG                         |   14     |   4   |   8   |   4

QB.REG                         |   15     |   8   |  10   |   2

QC.REG                         |   16     |   3   |  12   |   9

CA.REG                         |   21     |   2   |  12   |  10

CB.REG                         |   22     |   2   |  10   |   8

NEW_STEP.REG                   |   23     |   1   |   8   |   7

PHASE_4                        |   20     |   1   |  14   |  13

PHASE_3                        |   19     |   1   |  16   |  15

PHASE_2                        |   18     |   2   |  16   |  14

PHASE_1                        |   17     |   2   |  14   |  12

==== List of Inputs/Feedbacks ====

Signal Name                    | Pin      | Pin Type

============================== |==========|=========

CLOCK                          |    1     | CLK/IN

STEP                           |    2     | INPUT

DIR                            |    3     | INPUT

QA                             |   14     | BIDIR

QB                             |   15     | BIDIR

QC                             |   16     | BIDIR

CA                             |   21     | BIDIR

CB                             |   22     | BIDIR

NEW_STEP                       |   23     | BIDIR

Page 5

ABEL 4.30  -  Device Utilization Chart         Fri Apr 19 12:28:16 1996

Four phase stepper motor controller Type 3

==== P22V10 Unused Resources ====

Pin   |  Pin   |   Product   | Flip-flop

Number |  Type  |   Terms     |   Type

=======|========|=============|==========

4  |  INPUT |      -      |    -

5  |  INPUT |      -      |    -

6  |  INPUT |      -      |    -

7  |  INPUT |      -      |    -

8  |  INPUT |      -      |    -

9  |  INPUT |      -      |    -

10  |  INPUT |      -      |    -

11  |  INPUT |      -      |    -

13  |  INPUT |      -      |    -

Page 6

ABEL 4.30  -  Device Utilization Chart         Fri Apr 19 12:28:16 1996

Four phase stepper motor controller Type 3

==== I/O Files ====

Module: 'stepper_'

Input files

===========

ABEL PLA file: stepper_.tt3

Device library: P22V10.dev

Output files

============

Report file: stepper_.doc

Programmer load file: stepctrl.jed